Here are some of the signs, lights, and signals you will see as you drive along the Yonge-University-Spadina Subway:

# Static Signs

Operate the train in parallel mode (power position 3).

Operate the train in series mode (power position 2).

Place the train in coast mode (power position 0).

There are two of these in each station.  The first one provides a reference point for braking, the second represents the place where the train must be stopped.

These are left over from the past when trains had specific notches in their brake controls.  These are not used anymore, but can still be seen in some places, especially steep slops.

## Lights / Signals

Blue lights indicate the locations of an emergency power cut-off switches.  They are found at the ends of platforms and in the tunnels.

This is a three aspect automatic signal.  It is responsible for keeping trains safely spaced apart.  It can show red (stop), yellow (caution), and green (proceed).

This is a three aspect timed signal.  Each timed signal comes with a preset speed (represented in this route by T XX, where XX is the preset speed).  In the real world, this signal flashes red, but will clear to the display shown here if the signal is passed at or below the preset speed XX.  The preset speed must be maintained until passing the next signal.  In this route, most timed signals do not have the flashing red before the yellow over lunar due to the difficulty of programming this on curves or slopes.  Most only show the yellow over lunar, but the speed restriction remains.

This is a six aspect automatic signal.  The top head acts just as a three aspect automatic signal.  The bottom head indicates the presence of a diverging path.  If it is green, the primary (straight) route is available.  If it is showing yellow, a diverging route (such as a crossover) is available.  If it is showing red, no route is currently clear.

This is a six aspect timed signal.  It combines the functions of a six aspect automatic signal and a timed signal as explained above.

For more information on TTC signalling, visit this excellent site:

http://www3.sympatico.ca/gcircle/csbh/ttc/cover.html

© 2002 Dave Reage.

Signal aspect .bmps © 2002 by Jon Gulbis, used with permission.

If you have any questions or comments, please email me at d_reage@hotmail.com