Bombardier T1 driver’s manual

TABLE OF CONTENTS

Introduction

Train controls

Proper driving procedures

Diagnostics

Specifications

Acknowledgments

INTRODUCTION

The introduction to the Bombardier T1 can be found on the Transit Toronto website

TRAIN CONTROLS

Unlike it’s predecessors, the Bombardier T1 uses a T-handle joystick type controller for the throttle and brake.  There are 3 power positions, 3 brake positions and a “Coast” position

POWER positions:

- INCH

- SERIAL

- PARALLEL

The INCH position is used in instances when it is necessary to proceed with extreme caution:

- Going through a yard

- Driving through a “Point of sight” red signal, done only when instructed to do so by Transit Control

- Approaching and passing maintenance workers on the line, indicated by amber lights between the tracks in a tunnel, or yellow flags outside.  It is safe to accelerate when you pass the green light or flag at the end of the section occupied by the maintenance workers

The SERIAL position is used under normal circumstances when it is necessary to drive slowly:

- Driving through a switch, usually marked ahead of time with an “S” and/or a timed signal

- Going around a tight corner, usually marked ahead of time with an “S” and/or a timed signal

- Going through a yellow aspect

- Any area that is marked with an “S”

(You may proceed to accelerate when a green aspect or “P” is indicated)

The PARALLEL position is used under all other driving conditions.

BRAKE positions:

- MINNIMUM

- FULL

- EMERGENCY

(These are self-explanatory)

COAST position

The coast position is used to maintain a constant speed after acceleration.

EMERGENCY RESET

The EMERGENCY RESET button is used to reset the brakes after the emergency brakes have been tripped when going through a red aspect.

Diagram of the T1’s control panel indicating it’s major components:

PROPER DRIVING PROCEDURES

There are several bugs in the Bombardier T1 for BVE.  These are a result of making the train more realistic, but if you don’t follow the proper driving procedures, things will get a little messed up.

When stopped at a station and after the doors close, leave the controller in the BRAKE position.  If you put the controller into a POWER position, the DOOR SUPPLY light will go out prematurely and neither that light, nor the DOOR CLOSE light will be on until the doors fully close.  This is because on a real Bombardier T1, both lights stay on after the doors close and until the train starts moving, in case the guard needs to open the doors again before the train moves off.  After the DOOR SUPPLY light goes out, the guard is no longer able to open the doors.

Only power off the train, (Put the train into “PO” position) at the end of the route.  Powering off and powering back on the train will result in both DOOR SUPPLY and DOOR CLOSE lights remaining off after the train is powered back on, which is unrealistic, because the doors are either open or closed, resulting in one light or the other to be on, or in some instances, both (See above explanation).

DIAGNOSTICS

For those of you who are curious about what the lights and switches on the control panel are, here is a diagram:

(The EP BRAKE light always stays on as long as there is power fed to circuit that feeds the electro-pneumatic brakes, if this fails, the backup air brake system is used exclusively).

SPECIFICATIONS

Weight and capacity

Car weight (W1-tare): 33095 kg (72960 lb)

Car weight (W4-service): 50105 kg (110460 lb)

Car weight (W5-crush): 54527kg (120210 lb)

Seated passengers per car: 66

Standing passengers per car: 184

Crush load: 315

Dimensions

Length (over anti-climbers): 22698 mm (74' 5-5/8'')

Length (over coupler faces): 22787 mm (74' 9-1/8'')

Width (over side sheets): 3134 mm (10' 3-38'')

Maximum width: 3150 mm (10' 4'')

Height (rail to roof): 3658 mm (12' 0'')

Height (rail to top of floor): 1105 mm (43-1/2'')

Doorway width (side, clear opening): 1524 mm (5' 0'')

Doorway width (end, clear opening): 711 mm (28'')

Doorway height (side): 1930 mm (6' 4'')

Floor to ceiling height (high ceiling): 2184 mm (7' 2'')

Wheel diameter (new): 711 mm (28'')

Truck wheelbase: 2134 mm (7' 0'')

Truck centre distance: 16459 mm (54' 0'')

Track gauge: 1495 mm (58-7/8'')

Performance

Maximum design speed: 88 km/h (55 m.p.h)

Maximum operating speed: 80 km/h (50 m.p.h.)

Acceleration rate: 0.85 m/s2 (1.9 m.p.h.p.s)

Service braking: 1.30 m/s2 (2.9 m.p.h.p.s)

Emergency braking: 1.38 m/s2 (3.1 m.p.h.p.s)

Minimum lateral curve radius: 116 m (380')

Minimum vertical curve radius: 610 m (2000')

Electrical system

Power fed by third rail: 600 VDC

Auxiliary voltage: 120/208 VAC

Low voltage: 36.4 VDC

Interior lighting: Fluorescent

Miscellaneous

Carbody: Aluminum and steel

Propulsion system: AC motors

Truck type: Fabricated truck, M-A-N design

Number of trucks: 2

Primary suspension: Chevron elastrometric springs

Secondary suspension: Air springs

Braking: Pneumatic/rheostatic/regenerative (no brake pipe)

Heating: Overhead and baseboard heaters

Air conditioning: Split system, 12 tons

Wheelchair location: 1

Couplers: Automatic

Drawbars: Between cars of married pair

Monitoring system: On-board computer with diagnostic

ACKNOWLEDGMENTS

Thanks to Andrew Larson for providing the original photograph that was used in creating the graphics for the T1 control panel.

Thanks to Steve Green for letting me use some of the sounds from his Lt1995 London subway train.

Thanks to everyone else who had a part in helping create the Bombardier T1 for BVE.